VOCATIONAL TRAINING REPORT

Ena colliery Kusunda Area VI,

BCCL

By,

ANURAG KUMAR (19JEO170)

MINING ENGINEERING

IIT(ISM) DHANBAD

MINE TRAINING REPORT

DESIGN

- It is having big sump at the bottom.

- It is said that this mine was underground before and now it is made opencast.

- We saw XIV seam bench.

- Mine consisted of 5 benches.

- Height of bench is approximately 25m.

- Coal is of washery III, washery IV quality.

- It is a fiery seam i.e. strata temperature is greater than 80 deg Celsius.

- 100 tons dumper of caterpillar is used.

- Production blasting is done in the morning.

- Weighbridge is also installed in the mine.

- Crusher is also installed in the mine.

- Coal depo is also made in the mine.

- 2 Payloader was also present there.

- River is also located beside the mine at safe distance.

- SME(site mixed emulsion) is used as explosive.

BLASTING OPERATION

- SME (site mixed emulsion) is used as explosive. It is the mixture of emulsion, ANFO, Diseal oil, water, chemical it is prepared at the site itself.

- At the site 6m hole was created and 3.5m stemming was given.

- There was problem with some hole it was very hot greater than 80 degree celcius that hole temperature was brought down by pouring water. Temperature was measured using contactless thermometer.

- For hot hole DF(detonating fuse) was used inside it PETN mixture is layered over wire inner surface.

- Emulsion booster was used as it can withstand high temperature and it is suitable for hot hole blasting.

- In normal temperature hole i.e. below 80 degree Celsius NONEL along with cast booster is used. Cast booster contains solid high explosive.

- 20 holes was blasted, 13 normal holes and 7 hot temperature holes.

- Exploder operated by battery is used to initiate blasting.

- First cast booster/emulsion booster in sack is put down the hole.

- Then SME is poured inside the hole and then stemming is given.

- In stemming sand and fly ash was used.

- Hole diameter was 114 cm.

FIG-emulsion loading.

FIG- Cast booster used inside the hole.

SURVEYING

- Map is drawn on the scale of 1:2000.

- Total 14 seam is present in that area.

- Coal seam is determined by borehole drilling.

- Total lease area is 216 hectares(2.16km2).

- One river passes near from the open cast mines.

- 24m fault is present at the boundary in north side.

- At present work is going on in ix,x seam.

- Local residence is present near the mine planning is done to shift them to some other place.

- Two colliery is present in the west side.

- One colliery is also present at north side.

FIG- Rough map of the mine

FIG:- Surveying data of the ena colliery Area VI.

SAFETY MANAGEMENT PLAN

- Principle hazards are Dust generation which can cause pneumoconiosis.

- Subsidence of land can take place because as we are working on underground developed seam.

- To avoid this where safe distance(parting) between two contiguous seam is less than 6m first hole drilling, charging, blasting and compaction by dozer is done before working.

- Fly rocks is also a major problem for that muffle blasting is suggested.

- Lighting problem is also a issue in the mines and collision may take place during night shift.

PRODUCTION PLANNING

- 6m3 shovel is used in the mines.

- Per shovel 5 dumper is used in the mines.

- 26 100 tonne dumper is present in the mines.

FIG:- 100 tonne dumper